ITRF2020 proyección UTM 17 sur

PROJCS["ITRF2020_UTM_Zone_17S",

# GEOGCS["ITRF2020",

DATUM["International_Terrestrial_Reference_Frame_2020",

SPHEROID["GRS_1980",6378137.0,298.257222101]],

PRIMEM["Greenwich",0.0],

UNIT["Degree",0.0174532925199433]],

PROJECTION["Transverse_Mercator"],

PARAMETER["False_Easting",500000.0],

PARAMETER["False_Northing",10000000.0],

PARAMETER["Central_Meridian",-81.0],

PARAMETER["Scale_Factor",0.9996],

PARAMETER["Latitude_Of_Origin",0.0],

UNIT["Meter",1.0]]

ITRF2020 proyección UTM 17 norte

PROJCS["ITRF2020_UTM_Zone_17N",

GEOGCS["ITRF2020",

DATUM["International_Terrestrial_Reference_Frame_2020",

SPHEROID["GRS_1980",6378137.0,298.257222101]],

PRIMEM["Greenwich",0.0],

UNIT["Degree",0.0174532925199433]],

PROJECTION["Transverse_Mercator"],

PARAMETER["False_Easting",500000.0],

PARAMETER["False_Northing",0.0],

PARAMETER["Central_Meridian",-81.0],

PARAMETER["Scale_Factor",0.9996],

PARAMETER["Latitude_Of_Origin",0.0],

UNIT["Meter",1.0]]

ITRF2020 proyección UTM 18 sur

PROJCS["ITRF2020_UTM_Zone_18S",

GEOGCS["ITRF2020",

DATUM["International_Terrestrial_Reference_Frame_2020",

SPHEROID["GRS_1980",6378137.0,298.257222101]],

PRIMEM["Greenwich",0.0],

UNIT["Degree",0.0174532925199433]],

PROJECTION["Transverse_Mercator"],

PARAMETER["False_Easting",500000.0],

PARAMETER["False_Northing",10000000.0],

PARAMETER["Central_Meridian",-75.0],

PARAMETER["Scale_Factor",0.9996],

PARAMETER["Latitude_Of_Origin",0.0],

UNIT["Meter",1.0]]

ITRF2020 proyección UTM 18 norte

PROJCS["ITRF2020_UTM_Zone_18N",

GEOGCS["ITRF2020",

DATUM["International_Terrestrial_Reference_Frame_2020",

SPHEROID["GRS_1980",6378137.0,298.257222101]],

PRIMEM["Greenwich",0.0],

UNIT["Degree",0.0174532925199433]],

PROJECTION["Transverse_Mercator"],

PARAMETER["False_Easting",500000.0],

PARAMETER["False_Northing",0.0],

PARAMETER["Central_Meridian",-75.0],

PARAMETER["Scale_Factor",0.9996],

PARAMETER["Latitude_Of_Origin",0.0],

UNIT["Meter",1.0]]

ITRF2020 proyección UTM 15 sur

PROJCS["ITRF2020_UTM_Zone_15S",

GEOGCS["ITRF2020",

DATUM["International_Terrestrial_Reference_Frame_2020",

SPHEROID["GRS_1980",6378137.0,298.257222101]],

PRIMEM["Greenwich",0.0],

UNIT["Degree",0.0174532925199433]],

PROJECTION["Transverse_Mercator"],

PARAMETER["False_Easting",500000.0],

PARAMETER["False_Northing",10000000.0],

PARAMETER["Central_Meridian",-93.0],

PARAMETER["Scale_Factor",0.9996],

PARAMETER["Latitude_Of_Origin",0.0],

UNIT["Meter",1.0]]

ITRF2020 proyección UTM 15 norte

PROJCS["ITRF2020_UTM_Zone_15N",

GEOGCS["ITRF2020",

DATUM["International_Terrestrial_Reference_Frame_2020",

SPHEROID["GRS_1980",6378137.0,298.257222101]],

PRIMEM["Greenwich",0.0],

UNIT["Degree",0.0174532925199433]],

PROJECTION["Transverse_Mercator"],

PARAMETER["False_Easting",500000.0],

PARAMETER["False_Northing",0.0],

PARAMETER["Central_Meridian",-93.0],

PARAMETER["Scale_Factor",0.9996],

PARAMETER["Latitude_Of_Origin",0.0],

UNIT["Meter",1.0]]

ITRF2020 proyección UTM 16 sur

PROJCS["ITRF2020_UTM_Zone_16S",

GEOGCS["ITRF2020",

DATUM["International_Terrestrial_Reference_Frame_2020",

SPHEROID["GRS_1980",6378137.0,298.257222101]],

PRIMEM["Greenwich",0.0],

UNIT["Degree",0.0174532925199433]],

PROJECTION["Transverse_Mercator"],

PARAMETER["False_Easting",500000.0],

PARAMETER["False_Northing",10000000.0],

PARAMETER["Central_Meridian",-87.0],

PARAMETER["Scale_Factor",0.9996],

PARAMETER["Latitude_Of_Origin",0.0],

UNIT["Meter",1.0]]

ITRF2020 proyección UTM 16 norte

PROJCS["ITRF2020_UTM_Zone_16N",

GEOGCS["ITRF2020",

DATUM["International_Terrestrial_Reference_Frame_2020",

SPHEROID["GRS_1980",6378137.0,298.257222101]],

PRIMEM["Greenwich",0.0],

UNIT["Degree",0.0174532925199433]],

PROJECTION["Transverse_Mercator"],

PARAMETER["False_Easting",500000.0],

PARAMETER["False_Northing",0.0],

PARAMETER["Central_Meridian",-87.0],

PARAMETER["Scale_Factor",0.9996],

PARAMETER["Latitude_Of_Origin",0.0],

UNIT["Meter",1.0]]